# Torso-yaw repair

VOLUME =  2.3944790e+06  MM^3

SURFACE AREA =  1.3933974e+06  MM^2

AVERAGE DENSITY =  3.6366051e-06 KILOGRAM / MM^3

MASS =  8.7077745e+00 KILOGRAM

CENTER OF GRAVITY with respect to URDF_TORSO_YAW coordinate frame:

X   Y   Z     4.9776510e-01 -4.2604032e-01 -1.6838189e+02  MM

INERTIA with respect to URDF_TORSO_YAW coordinate frame:  (KILOGRAM * MM^2)

INERTIA TENSOR:

Ixx Ixy Ixz  3.6546671e+05  2.2248628e+01  7.4211840e+02

Iyx Iyy Iyz  2.2248628e+01  3.2762299e+05 -6.1767263e+02

Izx Izy Izz  7.4211840e+02 -6.1767263e+02  5.4103950e+04

INERTIA at CENTER OF GRAVITY with respect to URDF_TORSO_YAW coordinate frame:  (KILOGRAM * MM^2)

INERTIA TENSOR:

Ixx Ixy Ixz  1.1857829e+05  2.0401988e+01  1.2279511e+01

Iyx Iyy Iyz  2.0401988e+01  8.0733992e+04  7.0011268e+00

Izx Izy Izz  1.2279511e+01  7.0011268e+00  5.4100212e+04

PRINCIPAL MOMENTS OF INERTIA:  (KILOGRAM * MM^2)

I1  I2  I3   5.4100208e+04  8.0733983e+04  1.1857831e+05

ROTATION MATRIX from URDF_TORSO_YAW orientation to PRINCIPAL AXES:

-0.00019       -0.00054       -1.00000

-0.00026        1.00000       -0.00054

1.00000        0.00026       -0.00019

ROTATION ANGLES from URDF_TORSO_YAW orientation to PRINCIPAL AXES (degrees):

angles about x  y  z   0.000        -89.967          0.000

RADII OF GYRATION with respect to PRINCIPAL AXES:

R1  R2  R3 7.8821715e+01  9.6288540e+01  1.1669414e+02  MM

# Shoulder-pitch (right)

VOLUME =  5.5209105e+05  MM^3

SURFACE AREA =  4.0907966e+05  MM^2

AVERAGE DENSITY =  4.4205562e-06 KILOGRAM / MM^3

MASS =  2.4405495e+00 KILOGRAM

CENTER OF GRAVITY with respect to URDF_SH-PITCH_RIGHT coordinate frame:

X   Y   Z     1.0200651e+01 -2.1560998e+01  9.8767025e+01  MM

INERTIA with respect to URDF_SH-PITCH_RIGHT coordinate frame:  (KILOGRAM * MM^2)

INERTIA TENSOR:

Ixx Ixy Ixz  3.2134848e+04  6.1530839e+02 -2.8567365e+03

Iyx Iyy Iyz  6.1530839e+02  3.3532486e+04  6.4828934e+03

Izx Izy Izz -2.8567365e+03  6.4828934e+03  7.2415920e+03

INERTIA at CENTER OF GRAVITY with respect to URDF_SH-PITCH_RIGHT coordinate frame:  (KILOGRAM * MM^2)

INERTIA TENSOR:

Ixx Ixy Ixz  7.1929157e+03  7.8543184e+01 -3.9791236e+02

Iyx Iyy Iyz  7.8543184e+01  9.4711610e+03  1.2857051e+03

Izx Izy Izz -3.9791236e+02  1.2857051e+03  5.8530904e+03

PRINCIPAL MOMENTS OF INERTIA:  (KILOGRAM * MM^2)

I1  I2  I3   5.3544059e+03  7.2804526e+03  9.8823086e+03

ROTATION MATRIX from URDF_SH-PITCH_RIGHT orientation to PRINCIPAL AXES:

0.21414        0.97665       -0.01740

-0.29491        0.08162        0.95203

0.93122       -0.19874        0.30551

ROTATION ANGLES from URDF_SH-PITCH_RIGHT orientation to PRINCIPAL AXES (degrees):

angles about x  y  z -72.209         -0.997        -77.633

RADII OF GYRATION with respect to PRINCIPAL AXES:

R1  R2  R3 4.6839455e+01  5.4617948e+01  6.3633439e+01  MM

# T matrix “torso-yaw” to “sh-Pitch”

# Shoulder-roll

VOLUME =  3.0086752e+05  MM^3

SURFACE AREA =  1.7179152e+05  MM^2

AVERAGE DENSITY =  2.6165847e-06 KILOGRAM / MM^3

MASS =  7.8724534e-01 KILOGRAM

CENTER OF GRAVITY with respect to SH-ROLL_RIGHT coordinate frame:

X   Y   Z     4.5955243e-02 -4.4957128e+01 -3.0791013e+01  MM

INERTIA with respect to SH-ROLL_RIGHT coordinate frame:  (KILOGRAM * MM^2)

INERTIA TENSOR:

Ixx Ixy Ixz  5.1302696e+03 -3.8704542e-01 -2.5046297e-01

Iyx Iyy Iyz -3.8704542e-01  2.6752272e+03 -1.7566988e+03

Izx Izy Izz -2.5046297e-01 -1.7566988e+03  4.2544739e+03

INERTIA at CENTER OF GRAVITY with respect to SH-ROLL_RIGHT coordinate frame:  (KILOGRAM * MM^2)

INERTIA TENSOR:

Ixx Ixy Ixz  2.7927572e+03 -2.0135067e+00 -1.3644218e+00

Iyx Iyy Iyz -2.0135067e+00  1.9288488e+03 -6.6693438e+02

Izx Izy Izz -1.3644218e+00 -6.6693438e+02  2.6633366e+03

PRINCIPAL MOMENTS OF INERTIA:  (KILOGRAM * MM^2)

I1  I2  I3   1.5347277e+03  2.7927618e+03  3.0574531e+03

ROTATION MATRIX from SH-ROLL_RIGHT orientation to PRINCIPAL AXES:

0.00193       -1.00000       -0.00057

0.86091        0.00195       -0.50875

0.50875        0.00049        0.86092

ROTATION ANGLES from SH-ROLL_RIGHT orientation to PRINCIPAL AXES (degrees):

angles about x  y  z  30.580          0.000         89.889

RADII OF GYRATION with respect to PRINCIPAL AXES:

R1  R2  R3 4.4153041e+01  5.9560989e+01  6.2319628e+01  MM

# T matrix  “sh-pitch” to  “sh-roll”

# Shoulder-yaw

VOLUME =  2.9959881e+05  MM^3

SURFACE AREA =  2.3112453e+05  MM^2

AVERAGE DENSITY =  9.6290227e-06 KILOGRAM / MM^3

MASS =  2.8848438e+00 KILOGRAM

CENTER OF GRAVITY with respect to URDF_SH_YAW coordinate frame:

X   Y   Z     5.0993150e+00  5.9986523e+00 -1.8011742e+02  MM

INERTIA with respect to URDF_SH_YAW coordinate frame:  (KILOGRAM * MM^2)

INERTIA TENSOR:

Ixx Ixy Ixz  1.2499351e+05 -3.3982821e+02  5.0207852e+03

Iyx Iyy Iyz -3.3982821e+02  1.2562944e+05  4.9897675e+03

Izx Izy Izz  5.0207852e+03  4.9897675e+03  2.8120583e+03

INERTIA at CENTER OF GRAVITY with respect to URDF_SH_YAW coordinate frame:  (KILOGRAM * MM^2)

INERTIA TENSOR:

Ixx Ixy Ixz  3.1298785e+04 -2.5158367e+02  2.3711270e+03

Iyx Iyy Iyz -2.5158367e+02  3.1963501e+04  1.8728041e+03

Izx Izy Izz  2.3711270e+03  1.8728041e+03  2.6332359e+03

PRINCIPAL MOMENTS OF INERTIA:  (KILOGRAM * MM^2)

I1  I2  I3   2.3182985e+03  3.1477998e+04  3.2099226e+04

ROTATION MATRIX from URDF_SH_YAW orientation to PRINCIPAL AXES:

-0.08193        0.98321       -0.16305

-0.06353        0.15812        0.98537

0.99461        0.09109        0.04951

ROTATION ANGLES from URDF_SH_YAW orientation to PRINCIPAL AXES (degrees):

angles about x  y  z -87.124         -9.384        -94.763

RADII OF GYRATION with respect to PRINCIPAL AXES:

R1  R2  R3 2.8348072e+01  1.0445817e+02  1.0548389e+02  MM

T matrix “sh-roll” to “sh-yaw”

# Elbow

VOLUME =  2.3134175e+05  MM^3

SURFACE AREA =  1.3438238e+05  MM^2

AVERAGE DENSITY =  2.5998586e-06 KILOGRAM / MM^3

MASS =  6.0145584e-01 KILOGRAM

CENTER OF GRAVITY with respect to ELBOW coordinate frame:

X   Y   Z     3.3322750e-03 -3.3289421e+01 -2.3682302e+01  MM

INERTIA with respect to ELBOW coordinate frame:  (KILOGRAM * MM^2)

INERTIA TENSOR:

Ixx Ixy Ixz  2.6236849e+03 -1.1975532e+00 -1.1260155e+00

Iyx Iyy Iyz -1.1975532e+00  1.5640005e+03 -8.6948681e+02

Izx Izy Izz -1.1260155e+00 -8.6948681e+02  2.2746592e+03

INERTIA at CENTER OF GRAVITY with respect to ELBOW coordinate frame:  (KILOGRAM * MM^2)

INERTIA TENSOR:

Ixx Ixy Ixz  1.6198328e+03 -1.2642724e+00 -1.1734799e+00

Iyx Iyy Iyz -1.2642724e+00  1.2266731e+03 -3.9531701e+02

Izx Izy Izz -1.1734799e+00 -3.9531701e+02  1.6081346e+03

PRINCIPAL MOMENTS OF INERTIA:  (KILOGRAM * MM^2)

I1  I2  I3   9.7847599e+02  1.6198369e+03  1.8563277e+03

ROTATION MATRIX from ELBOW orientation to PRINCIPAL AXES:

0.00264       -1.00000       -0.00136

0.84692        0.00296       -0.53172

0.53172        0.00025        0.84692

ROTATION ANGLES from ELBOW orientation to PRINCIPAL AXES (degrees):

angles about x  y  z  32.122         -0.078         89.849

RADII OF GYRATION with respect to PRINCIPAL AXES:

R1  R2  R3 4.0334178e+01  5.1895986e+01  5.5555294e+01  MM

# Tmatrix “sh-yaw” to “Elbow”

# Forearm –yaw

VOLUME =  3.9859448e+05  MM^3

SURFACE AREA =  2.7083054e+05  MM^2

AVERAGE DENSITY =  3.2779068e-06 KILOGRAM / MM^3

MASS =  1.3065556e+00 KILOGRAM

CENTER OF GRAVITY with respect to URDF_FOREARM-YAW coordinate frame:

X   Y   Z     2.0004978e+00 -5.9178127e+01 -1.5065406e+02  MM

INERTIA with respect to URDF_FOREARM-YAW coordinate frame:  (KILOGRAM * MM^2)

INERTIA TENSOR:

Ixx Ixy Ixz  4.4132328e+04  1.9917841e+02  4.0712797e+02

Iyx Iyy Iyz  1.9917841e+02  3.6835642e+04 -1.4836035e+04

Izx Izy Izz  4.0712797e+02 -1.4836035e+04  9.1610630e+03

INERTIA at CENTER OF GRAVITY with respect to URDF_FOREARM-YAW coordinate frame:  (KILOGRAM * MM^2)

INERTIA TENSOR:

Ixx Ixy Ixz  9.9022766e+03  4.4500899e+01  1.3354192e+01

Iyx Iyy Iyz  4.4500899e+01  7.1759859e+03 -3.1875374e+03

Izx Izy Izz  1.3354192e+01 -3.1875374e+03  4.5802104e+03

PRINCIPAL MOMENTS OF INERTIA:  (KILOGRAM * MM^2)

I1  I2  I3   2.4362822e+03  9.3182543e+03  9.9039363e+03

ROTATION MATRIX from URDF_FOREARM-YAW orientation to PRINCIPAL AXES:

-0.00481       -0.05039       -0.99872

0.55808        0.82859       -0.04449

0.82977       -0.55758        0.02413

ROTATION ANGLES from URDF_FOREARM-YAW orientation to PRINCIPAL AXES (degrees):

angles about x  y  z  61.523        -87.099         95.454

RADII OF GYRATION with respect to PRINCIPAL AXES:

R1  R2  R3 4.3181712e+01  8.4450716e+01  8.7064273e+01  MM

# T matrix “elbow” to “forearm-yaw”

# Wrist-pitch

VOLUME =  3.6923988e+05  MM^3

SURFACE AREA =  2.3019064e+05  MM^2

AVERAGE DENSITY =  3.1288214e-06 KILOGRAM / MM^3

MASS =  1.1552856e+00 KILOGRAM

CENTER OF GRAVITY with respect to URDF_WRIST-PITCH coordinate frame:

X   Y   Z    -5.8850122e+00 -2.0700011e+01  4.4606499e+01  MM

INERTIA with respect to URDF_WRIST-PITCH coordinate frame:  (KILOGRAM * MM^2)

INERTIA TENSOR:

Ixx Ixy Ixz  6.1066940e+03 -3.6875118e+02  1.8837192e+02

Iyx Iyy Iyz -3.6875118e+02  5.3282000e+03  1.0176711e+03

Izx Izy Izz  1.8837192e+02  1.0176711e+03  4.2216013e+03

INERTIA at CENTER OF GRAVITY with respect to URDF_WRIST-PITCH coordinate frame:  (KILOGRAM * MM^2)

INERTIA TENSOR:

Ixx Ixy Ixz  3.3129474e+03 -2.2801449e+02 -1.1490187e+02

Iyx Iyy Iyz -2.2801449e+02  2.9894708e+03 -4.9067763e+01

Izx Izy Izz -1.1490187e+02 -4.9067763e+01  3.6865610e+03

PRINCIPAL MOMENTS OF INERTIA:  (KILOGRAM * MM^2)

I1  I2  I3   2.8602712e+03  3.4092743e+03  3.7194336e+03

ROTATION MATRIX from URDF_WRIST-PITCH orientation to PRINCIPAL AXES:

0.47035       -0.83532       -0.28463

0.87465        0.48414        0.02449

0.11734       -0.26047        0.95833

ROTATION ANGLES from URDF_WRIST-PITCH orientation to PRINCIPAL AXES (degrees):

angles about x  y  z  -1.464        -16.537         60.617

RADII OF GYRATION with respect to PRINCIPAL AXES:

R1  R2  R3 4.9757542e+01  5.4323318e+01  5.6740573e+01  MM

# T matrix “forearm-yaw” to “wrist-pitch”

# Urdf wrist-roll

VOLUME =  5.0405586e+04  MM^3

SURFACE AREA =  2.1012175e+04  MM^2

AVERAGE DENSITY =  2.8100000e-06 KILOGRAM / MM^3

MASS =  1.4163970e-01 KILOGRAM

CENTER OF GRAVITY with respect to URDF_WRIST-ROLL coordinate frame:

X   Y   Z     0.0000000e+00 -1.0491951e+00 -1.0307299e+01  MM

INERTIA with respect to URDF_WRIST-ROLL coordinate frame:  (KILOGRAM * MM^2)

INERTIA TENSOR:

Ixx Ixy Ixz  7.6683110e+01 -7.3712424e-05  0.0000000e+00

Iyx Iyy Iyz -7.3712423e-05  7.9295379e+01 -1.0538792e+00

Izx Izy Izz  0.0000000e+00 -1.0538792e+00  1.1895336e+02

INERTIA at CENTER OF GRAVITY with respect to URDF_WRIST-ROLL coordinate frame:  (KILOGRAM * MM^2)

INERTIA TENSOR:

Ixx Ixy Ixz  6.1479333e+01 -7.4023399e-05  0.0000000e+00

Iyx Iyy Iyz -7.4023397e-05  6.4247521e+01  4.7786448e-01

Izx Izy Izz  0.0000000e+00  4.7786448e-01  1.1879744e+02

PRINCIPAL MOMENTS OF INERTIA:  (KILOGRAM * MM^2)

I1  I2  I3   6.1479333e+01  6.4243335e+01  1.1880163e+02

ROTATION MATRIX from URDF_WRIST-ROLL orientation to PRINCIPAL AXES:

1.00000       -0.00003        0.00000

0.00003        0.99996        0.00876

0.00000       -0.00876        0.99996

ROTATION ANGLES from URDF_WRIST-ROLL orientation to PRINCIPAL AXES (degrees):

angles about x  y  z  -0.502          0.000          0.000

RADII OF GYRATION with respect to PRINCIPAL AXES:

R1  R2  R3 2.0833972e+01  2.1297153e+01  2.8961343e+01  MM

# T matrix “wrist-pitch” to “wrist_roll”